FINAL PROJECT REPORT

Locality-Level Air and Noise Pollution Mapping and Automated Surveillance Dashboard

•	
      Document Type: Final Academic Field Project Report

•	
      Course / Subject: Core Engineering / Environmental Informatics Field Project

•	
      Candidate Name: Manav Singh

•	
      Repository: https://github.com/manavssingh/Pollution-Mapper

TABLE OF CONTENTS

1	
      Abstract

2	
      Chapter 1: Problem Statement & Motivation

3	
      Chapter 2: Literature Review

4	
      Chapter 3: Methodology & System Architecture

5	
      Chapter 4: Data Collection & Empirical Analysis

6	
      Chapter 5: Conclusions

7	
      Chapter 6: Actionable Recommendations

8	
      References

ABSTRACT

Urban atmospheric pollution and environmental acoustic noise represent two of the most critical anthropogenic threats to human health in modern developing metropolises. While atmospheric particulate matter ($PM_{2.5}, PM_{10}$) directly impairs cardiopulmonary function, excessive acoustic noise exposure ($> 55\text{ dB}$) causes chronic sleep disturbance, cognitive impairment, neuro-endocrine stress, and cardiovascular diseases.

This final report presents the design, field implementation, and analytical findings of Pollution-Mapper, a serverless, locality-level multi-pollutant surveillance dashboard. Addressing the lack of open public APIs for street noise, the study deployed a hybrid data methodology: combining primary, on-site decibel meter measurements ($L_{\text{Aeq}}$) across six diverse urban land-use typologies with automated, secondary air quality monitoring feeds (WAQI and Open-Meteo REST APIs). The platform features dynamic global geocoding, simultaneous dual-pollutant GIS mapping, an autonomous daily GitHub Actions CI/CD data pipeline, and empirical Pearson correlation analysis ($r = +0.68$), proving the co-location of severe acoustic and particulate exposure along major transit corridors.

CHAPTER 1: PROBLEM STATEMENT & MOTIVATION

1.1 Problem Statement

Conventional municipal environmental surveillance systems suffer from three fundamental deficiencies: 1. The Spatial Granularity Gap: State and national monitoring towers are spaced miles apart, providing macroscopic regional averages that conceal extreme microclimatic exposure disparities between congested transit junctions and adjacent residential buffers. 2. The Acoustic Monitoring Blindspot: Unlike ambient air pollution, continuous urban acoustic data is completely absent from open public internet APIs due to privacy concerns surrounding live street microphones and high sensor deployment costs. 3. Data Siloing: Atmospheric pollution and acoustic noise are treated as independent phenomena by public health agencies, despite sharing common vehicular and industrial origins.

1.2 Motivation & Objectives

The core motivation of this field project is to build an open-access, zero-cost, serverless platform enabling citizens, researchers, and urban administrators to visualize, map, and correlate hyper-local air and noise pollution levels. * Objective 1: Conduct on-site empirical acoustic surveys across representative urban land-use categories. * Objective 2: Automate real-time secondary air quality data ingestion from global station networks. * Objective 3: Engineer an interactive GIS web interface using Streamlit and Plotly MapLibre. * Objective 4: Establish an autonomous, serverless cloud data pipeline via GitHub Actions to accumulate historical trend data without operational expenditure. * Objective 5: Statistically evaluate the co-location of vehicular noise and particulate concentrations.

CHAPTER 2: LITERATURE REVIEW

2.1 Environmental Health Frameworks

•	
      World Health Organization (WHO) Noise Guidelines (2018): Establishes a conditional recommendation that daytime outdoor residential noise should not exceed $55\text{ dBA}$ $L_{\text{den}}$, and nighttime noise should remain below $40\text{ dBA}$ to prevent ischemic heart disease and severe sleep disturbance.

•	
      US-EPA Air Quality Index Standard (2018): Categorizes particulate matter ($PM_{2.5}$) into health-based color tiers: Good (0–50), Moderate (51–100), Unhealthy for Sensitive Groups (101–150), Unhealthy (151–200), Very Unhealthy (201–300), and Hazardous (301+).

2.2 Existing Systems Comparative Analysis

Platform

Spatial Resolution

Pollutants Covered

Acoustic Integration

Cloud Architecture

Maintenance Cost

Government CPCB / EPA

Macroscopic / Regional

$PM_{2.5}, PM_{10}, SO_2$

❌ None

Dedicated Government Servers

High ($>\$50,000/\text{station}$)

WAQI / IQAir Portals

City-Wide Stations

Composite AQI, PM

❌ None

Monolithic Cloud Clusters

High Infrastructure

Proposed Pollution-Mapper

Street / Micro-Locality

AQI, $PM_{2.5}, PM_{10}$, Noise (dB)

✅ Primary & Baseline

Serverless Git-Backed Engine

Zero Cost

CHAPTER 3: METHODOLOGY & SYSTEM ARCHITECTURE

3.1 The Hybrid Data Model

flowchart TD

subgraph Data Acquisition

API["Secondary Automated Feeds (WAQI & Open-Meteo REST APIs)"]

Field["Primary Field Survey (Calibrated Sound Level Meter Apps)"]

Model["Diurnal Urban Traffic Acoustic Model (WHO Guidelines)"]

end

subgraph Data Pipeline & Storage

StateEngine["Ingestion & State Controller"]

CSV_Air[("data/air_quality.csv")]

CSV_Noise[("data/noise_readings.csv")]

end

subgraph Serverless CI/CD Automation

GHA["GitHub Actions Scheduled Runner (Daily 06:00 UTC)"]

GHS["GitHub Secrets (WAQI_TOKEN)"]

end

subgraph Presentation & Analytics Layer

UI["Streamlit Web Dashboard (app.py)"]

Map["Plotly MapLibre Vector GIS Map"]

Corr["Pearson Correlation Analytics Engine"]

end

API --> StateEngine

Field --> StateEngine

Model --> StateEngine

StateEngine --> CSV_Air

StateEngine --> CSV_Noise

GHS --> GHA

GHA -->|Automated Daily Run| StateEngine

CSV_Air --> UI

CSV_Noise --> UI

UI --> Map

UI --> Corr

1	
      Secondary Data (Air Quality): Automated RESTful HTTP GET queries pull real-time $PM_{2.5}, PM_{10}$, and $US\ AQI$ values from monitoring stations.

2	
      Primary Data (Acoustic Field): Field measurements conducted using IEC 61672-1 Type 2 configured smartphone sound meters (A-weighting, slow response, 60-second $L_{\text{Aeq}}$ integration).

3	
      Modeled Urban Baseline: For global searched cities lacking field surveys, a diurnal traffic acoustic model synthesizes contextual ambient baselines, maintaining transparent data tagging in the user interface.

CHAPTER 4: DATA COLLECTION & EMPIRICAL ANALYSIS

4.1 Field Survey Site Measurements

Field sampling was conducted across six distinct urban land-use typologies:

Site

Locality

Land-Use Typology

Sound Level ($L_{\text{Aeq}}$)

AQI

WHO Delta

Health Status

S1

Shivajinagar Junction

Major Transit Terminal

82.4 dB

92

$+27.4\text{ dB}$

Moderate

S2

Laxmi Road

Commercial Retail Market

78.0 dB

88

$+23.0\text{ dB}$

Moderate

S3

Pashan Nature Reserve

Urban Green Buffer

45.2 dB

48

$-9.8\text{ dB}$

Good

S4

Katraj Colony

Residential Suburb

49.5 dB

58

$-5.5\text{ dB}$

Moderate

S5

Hadapsar Bypass

Freight Highway Corridor

85.1 dB

124

$+30.1\text{ dB}$

Unhealthy (Sensitive)

S6

Bhosari Cluster

Industrial Manufacturing

88.0 dB

155

$+33.0\text{ dB}$

Unhealthy

4.2 Statistical Correlation Analysis

Merging co-located atmospheric and acoustic observations reveals a strong positive Pearson Correlation Coefficient ($r = +0.68$): $$r = \frac{\sum (x_i - \bar{x})(y_i - \bar{y})}{\sqrt{\sum (x_i - \bar{x})^2 \sum (y_i - \bar{y})^2}}$$

Interpretation: Arterial vehicular traffic acts as a common driving variable: heavy diesel vehicle acceleration and continuous horn honking simultaneously drive acoustic noise above $80\text{ dB}$ and elevate particulate matter concentrations above $40\ \mu\text{g/m}^3$.

4.3 Qualitative Stakeholder Feedback

Interviews with 15 on-site stakeholders (traffic police constables, commercial vendors, pedestrians) revealed: * $100\%$ of surveyed traffic personnel reported chronic auditory fatigue and headaches. * Over $60\%$ of pedestrians identified horn honking—rather than engine noise—as the primary psychological stressor. * Tree buffers in green zones were widely acknowledged for reducing both perceived heat and acoustic strain.

CHAPTER 5: CONCLUSIONS

1	
      Severe Urban Non-Compliance: Four out of the six surveyed urban zones exceeded the WHO outdoor daytime noise limit of $55\text{ dB}$, with commercial and freight corridors exceeding safe baselines by over $25\text{ dB}$.

2	
      Coupled Environmental Penalty: The positive correlation ($r = +0.68$) demonstrates that urban commuters and outdoor workers face simultaneous high-level acoustic and particulate exposure in transportation hubs.

3	
      Attenuation by Natural Infrastructure: Natural green buffers (e.g., Pashan Sanctuary) attenuated ambient noise by over $30\text{ dB}$ and reduced particulate matter by $60\%$ compared to nearby highway corridors.

4	
      Feasibility of Serverless Surveillance: The project proved that low-cost field instrumentation combined with Git-backed storage and GitHub Actions CI/CD automation can deliver continuous, production-grade environmental surveillance at zero server cost.

CHAPTER 6: ACTIONABLE RECOMMENDATIONS

6.1 Urban Engineering & Policy Interventions

1	
      Deployment of Vegetative Acoustic Barriers: Plant dense, multi-layered native evergreen tree canopies along arterial flyovers and bypass roads to absorb sound energy and filter particulate matter.

2	
      Automated Acoustic Camera Enforcement: Install directional microphone arrays connected to license-plate recognition cameras at major intersections to penalize vehicle horn abuse.

3	
      Pedestrianization of Commercial Corridors: Implement weekend vehicle restrictions on historic market high-streets (e.g., Laxmi Road) to eliminate diesel idling and street canyon acoustic reverberation.

6.2 Occupational Health Measures

1	
      Mandatory Protective Gear for Traffic Personnel: Provide certified N95 respirators and acoustic earplugs for traffic police officers stationed at high-exposure transit nodes.

2	
      Rotational Shift Scheduling: Restrict continuous duty at high-noise intersections to maximum 4-hour shifts, followed by rotation to lower-exposure beats.

REFERENCES

1	
      World Health Organization (WHO), "Environmental Noise Guidelines for the European Region", Copenhagen, 2018.

2	
      United States Environmental Protection Agency (US-EPA), "Technical Assistance Document for Reporting Daily Air Quality (AQI)", EPA-454/B-18-007, 2018.

3	
      World Air Quality Index Project, "WAQI Data Platform Documentation", https://aqicn.org/api/.

4	
      Open-Meteo, "Global Air Quality API Documentation", https://open-meteo.com/en/docs/air-quality-api.